🤖 AUTONOMOUS FLOOR ROBOT Full Workflow Plan & Technical Documentation ROS 2 Jazzy • Ubuntu Server 24.04 • RADXA ROCK 4C+

| Hardware Stack • RADXA ROCK 4C+ (ROS 2 SBC) • ODrive v3.6 + Hoverboard wheels • RPLidar / YDLidar (360° LiDAR) • USB Camera • ESP32 (micro-ROS sensor bridge) • Compass / IMU + Ultrasonic | Software Stack • Ubuntu Server 24.04 (headless) • ROS 2 Jazzy (LTS) • SLAM Toolbox • Navigation2 (Nav2) • rosbridge_suite (WebSocket) • FastAPI + React Web Dashboard • micro-ROS Agent (ESP32 bridge) |
| --- | --- |

This document covers all 10 development phases, from hardware bringup through autonomous navigation, web dashboard, and safety systems. Follow phases in order — each phase validates the one before it.

SECTION 1 — ROBOT OVERVIEW & GOALS

## Robot Goal

Build a ROS2 autonomous mobile robot with capabilities similar to a vacuum cleaner robot, indoor delivery robot, and waypoint navigation robot.

| Capability | Description | Status |
| --- | --- | --- |
| Autonomous movement | Navigate rooms without human input | Target |
| Obstacle avoidance | Detect and route around obstacles in real time | Target |
| Waypoint navigation | Move to user-defined points (Point1, Point2, etc.) | Target |
| Return home | Navigate back to charging dock automatically | Target |
| Web dashboard | Remote control and live monitoring via browser | Target |
| Live monitoring | Camera stream, battery, position, motor status | Target |
| Autonomous patrol | Loop through waypoints on schedule | Future |

SECTION 2 — HARDWARE ARCHITECTURE

## Hardware Components

### Main Brain — RADXA ROCK 4C+

| Item | Detail |
| --- | --- |
| SBC | RADXA ROCK 4C+ (4GB RAM) |
| CPU | Rockchip RK3399-T, ARM Cortex-A72 + A53 |
| OS | Ubuntu Server 24.04 (headless) |
| ROS | ROS 2 Jazzy |
| Runs | Navigation stack, SLAM, dashboard backend, all ROS nodes |

### Motor System — ODrive + Hoverboard Wheels

| Item | Detail |
| --- | --- |
| Controller | ODrive v3.6 (USB connection to ROCK 4C+) |
| Motors | Hoverboard BLDC motors (×2) — differential drive |
| Power | 36V battery |
| Feedback | Encoder feedback for odometry |
| ROS driver | odrive_ros2 (build from source, Jazzy branch) |

### Sensor System

| Sensor | Interface | ROS Topic | Primary Use |
| --- | --- | --- | --- |
| LiDAR (RPLidar/YDLidar) | USB → /dev/ttyUSB0 | /scan | Mapping, localization, obstacle avoidance |
| Camera | USB / CSI | /image_raw | Live monitoring, future AI vision |
| Compass / IMU | I²C → ESP32 → USB | /imu/data | Heading estimation, EKF fusion |
| Ultrasonic sensors | GPIO → ESP32 → USB | /ultrasonic | Short-range obstacle / cliff detection |
| RGB LED | GPIO → ESP32 | N/A (output) | Status indicator |

### ESP32 — Sensor Bridge

The ESP32 acts as a low-level sensor controller, running micro-ROS firmware. It bridges I²C and GPIO sensors into the ROS 2 DDS graph over USB serial.

# Run micro-ROS agent on ROCK 4C+

ros2 run micro_ros_agent micro_ros_agent serial --dev /dev/ttyUSB1 -b 115200

### Power System

| Device | Voltage | Source |
| --- | --- | --- |
| Hoverboard motors | 36V | 36V LiPo / lead-acid battery |
| ODrive v3.6 | 36V | 36V battery |
| RADXA ROCK 4C+ | 5V | DC-DC step-down regulator from 36V |
| ESP32 | 5V | USB from ROCK 4C+ or separate regulator |
| LiDAR | 5V | USB from ROCK 4C+ |
| Camera | 5V | USB from ROCK 4C+ |

### USB Device Mapping (udev rules)

Add to /etc/udev/rules.d/99-robot.rules to prevent port swapping on reboot:

SUBSYSTEM=="tty", ATTRS{idVendor}=="10c4", SYMLINK+="lidar"

SUBSYSTEM=="tty", ATTRS{idVendor}=="1209", SYMLINK+="odrive"

SUBSYSTEM=="tty", ATTRS{idVendor}=="10c4", ATTRS{idProduct}=="ea60", SYMLINK+="esp32"

This gives stable device names: /dev/lidar, /dev/odrive, /dev/esp32

SECTION 3 — SOFTWARE ARCHITECTURE

## System Architecture Overview

The full system uses a layered architecture. The web dashboard communicates over WebSocket (rosbridge). The ROCK 4C+ runs all ROS 2 nodes. Hardware interfaces via USB to ODrive and ESP32.

Web Dashboard (React / HTML)

rosbridge_suite — WebSocket :9090

ROS 2 Jazzy Node Layer (ROCK 4C+)

lidar_node /scan; camera_node /image_raw; slam_toolbox /map; navigation2 Nav2. waypoint_mgr; obstacle_avoid; odrive_node /cmd_vel; esp32_bridge micro-ROS.

ODrive v3.6 → Hoverboard wheels (×2); ESP32 → Compass / Ultrasonic / RGB.

## ROS 2 Package List

| Package | Install / Source | Role |
| --- | --- | --- |
| rplidar_ros / sllidar_ros2 | sudo apt install ros-jazzy-rplidar-ros | LiDAR driver → /scan |
| v4l2_camera or usb_cam | sudo apt install ros-jazzy-usb-cam | Camera → /image_raw |
| slam_toolbox | sudo apt install ros-jazzy-slam-toolbox | Online SLAM → /map |
| nav2_bringup | sudo apt install ros-jazzy-nav2-bringup | Full Nav2 stack |
| rosbridge_suite | sudo apt install ros-jazzy-rosbridge-suite | WebSocket for dashboard |
| odrive_ros2 | Build from source (Jazzy branch) | ODrive ↔ /cmd_vel, /odom |
| micro_ros_agent | Build from source (micro_ros_setup) | ESP32 serial bridge |
| robot_localization | sudo apt install ros-jazzy-robot-localization | EKF: odom + IMU fusion |
| teleop_twist_keyboard | sudo apt install ros-jazzy-teleop-twist-keyboard | Manual keyboard control |

SECTION 4 — DEVELOPMENT PHASES

Follow these 10 phases in order. Each phase validates the previous. Never skip ahead — working motion at every stage makes debugging much easier.

| Phase 01 System Setup | Goal: Install OS, ROS 2, SSH access, test all USB devices |
| --- | --- |
|  | 1. Flash Ubuntu Server 24.04 onto ROCK 4C+ SD card / eMMC |
|  | 2. Install ROS 2 Jazzy: sudo apt install ros-jazzy-ros-base |
|  | 3. Configure SSH for headless operation |
|  | 4. Run lsusb to verify LiDAR, Camera, ODrive, ESP32 are detected |
|  | 5. Add udev rules to fix device port names (/dev/lidar, /dev/odrive, /dev/esp32) |
|  | 6. Set up auto-start ROS nodes on boot via systemd services |

| Phase 02 Motor Control | Goal: Get wheels spinning, verify odometry over ROS |
| --- | --- |
|  | 1. Wire hoverboard motors to ODrive motor phases |
|  | 2. Connect ODrive to ROCK 4C+ via USB |
|  | 3. Run ODrive calibration: odrivetool, then odrv0.axis0.requested_state = AXIS_STATE_FULL_CALIBRATION_SEQUENCE |
|  | 4. Set velocity control mode: odrv0.axis0.controller.config.control_mode = CONTROL_MODE_VELOCITY_CONTROL |
|  | 5. Install and build odrive_ros2 package from source |
|  | 6. Test: publish /cmd_vel and verify wheels respond correctly |
|  | 7. Confirm /odom topic appears with encoder feedback |

| Phase 03 Sensor Integration | Goal: All sensors publishing ROS topics |
| --- | --- |
|  | 1. Install LiDAR driver: sudo apt install ros-jazzy-rplidar-ros |
|  | 2. Launch LiDAR node and verify: ros2 topic echo /scan |
|  | 3. Install camera driver and verify /image_raw publishes |
|  | 4. Flash ESP32 with micro-ROS firmware (Arduino + micro_ros_arduino library) |
|  | 5. Start micro-ROS agent: ros2 run micro_ros_agent micro_ros_agent serial --dev /dev/esp32 |
|  | 6. Verify /imu/data (compass) and /ultrasonic topics appear |
|  | 7. Set up robot_localization EKF to fuse /odom + /imu/data → /odom/filtered |

| Phase 04 Robot URDF Model | Goal: Create accurate robot description for TF transforms |
| --- | --- |
|  | 1. Create a ROS 2 package: ros2 pkg create my_robot_description |
|  | 2. Write URDF/Xacro file defining: base_link, left_wheel, right_wheel joints |
|  | 3. Define sensor positions: LiDAR mount (top center), camera mount (front) |
|  | 4. Set accurate wheel_separation and wheel_radius (measure physical robot) |
|  | 5. Publish TF tree: base_link → laser, base_link → camera_link |
|  | 6. Verify in RViz2: ros2 launch my_robot_description display.launch.py |

| Phase 05 Teleoperation | Goal: Drive robot manually before any autonomous operation |
| --- | --- |
|  | 1. Install: sudo apt install ros-jazzy-teleop-twist-keyboard |
|  | 2. Launch: ros2 run teleop_twist_keyboard teleop_twist_keyboard |
|  | 3. Drive robot forward, backward, rotate left and right |
|  | 4. Verify /cmd_vel commands reach ODrive and wheels respond |
|  | 5. Tune max linear and angular velocities in ODrive config |
|  | 6. This phase confirms the full motion chain is working end-to-end |

| Phase 06 SLAM Mapping | Goal: Create a 2D occupancy grid map of the room |
| --- | --- |
|  | 1. Install: sudo apt install ros-jazzy-slam-toolbox |
|  | 2. Launch SLAM in online async mode: ros2 launch slam_toolbox online_async_launch.py |
|  | 3. Drive robot manually through the entire room using teleop |
|  | 4. Watch map build in RViz2 (/map topic) |
|  | 5. When room is fully covered, save the map: |
|  | 6. ros2 run nav2_map_server map_saver_cli -f ~/maps/room1 |
|  | 7. This creates room1.pgm (image) + room1.yaml (metadata) |

| Phase 07 Localization | Goal: Robot knows its position within the saved map |
| --- | --- |
|  | 1. Install AMCL (part of Nav2): sudo apt install ros-jazzy-nav2-bringup |
|  | 2. Configure nav2_params.yaml: set map path, robot footprint, scan topic |
|  | 3. Launch Nav2 in localization mode with saved map |
|  | 4. Set initial pose in RViz2 using '2D Pose Estimate' tool |
|  | 5. Move robot manually — verify pose estimate stays accurate on the map |
|  | 6. Compass (via EKF) helps maintain heading when wheels slip on smooth floors |

| Phase 08 Autonomous Navigation | Goal: Robot moves to goals automatically, avoids obstacles |
| --- | --- |
|  | 1. Launch full Nav2 stack with AMCL localization |
|  | 2. Send a navigation goal via RViz2 '2D Nav Goal' tool |
|  | 3. Verify robot plans a path and drives to the goal autonomously |
|  | 4. Test obstacle avoidance: place an object in the path, verify re-route |
|  | 5. Configure coverage planner for room-sweeping behavior (boustrophedon) |
|  | 6. Test 'Return Home': store home pose, trigger return-to-home action |
|  | 7. Set up Nav2 waypoint follower: ros2 run nav2_waypoint_follower waypoint_follower |

| Phase 09 Web Dashboard | Goal: Control and monitor robot from any browser |
| --- | --- |
|  | 1. Install rosbridge: sudo apt install ros-jazzy-rosbridge-suite |
|  | 2. Launch WebSocket server: ros2 launch rosbridge_server rosbridge_websocket_launch.xml |
|  | 3. Build FastAPI backend with REST endpoints for mission control |
|  | 4. Build React (or HTML) frontend using roslibjs for live ROS topic subscription |
|  | 5. Implement live map display (subscribe /map), robot pose overlay (/amcl_pose) |
|  | 6. Implement camera stream (MJPEG from /image_raw via web_video_server) |
|  | 7. Add control buttons: Start, Stop, Emergency Stop, Go Point1/2/3, Return Home |
|  | 8. Add live status panel: battery %, motor RPM, LiDAR status, current state |

| Phase 10 Safety System | Goal: Hardware and software emergency stop — CRITICAL |
| --- | --- |
|  | 1. ESP32 monitors ultrasonic sensors continuously (independent of ROS) |
|  | 2. If obstacle closer than threshold: ESP32 sends STOP command to ODrive immediately via UART |
|  | 3. This hardware-level stop works even if ROCK 4C+ crashes or ROS node dies |
|  | 4. Add software emergency stop: Nav2 costmap inflation layer from /scan |
|  | 5. Add physical emergency stop button wired to ESP32 GPIO |
|  | 6. Implement low-battery detection: ODrive reports bus voltage → trigger return-home |
|  | 7. Test all safety systems independently before deploying autonomously |

SECTION 5 — WAYPOINT NAVIGATION WORKFLOW

## Waypoint Mission Flow

The following workflow describes what happens from the moment a user clicks a waypoint on the dashboard to when the robot arrives.

| 1 | User clicks 'Go to Point 1' on web dashboard |
| --- | --- |
| 2 | Dashboard sends goal via rosbridge WebSocket to /navigate_to_pose action |
| 3 | Nav2 BT Navigator receives the action goal |
| 4 | Global planner (NavFn/Smac) computes optimal path on occupancy map |
| 5 | DWB local controller publishes /cmd_vel to follow the path |
| 6 | LiDAR scan updates costmap — obstacles inflate robot clearance zone |
| 7 | If obstacle blocks path: local planner re-routes around it in real time |
| 8 | Robot arrives at Point 1 — Nav2 reports SUCCESS to dashboard |
| 9 | Dashboard updates status: 'Arrived at Point 1' |

## Return Home Feature

The robot stores the home pose (dock position) at startup. When triggered — either by low battery or dashboard command — it executes a Nav2 navigate_to_pose goal to the home pose.

# Save home pose at startup (in launch file or param)

home_pose: {x: 0.0, y: 0.0, theta: 0.0}  # map frame

For a physical charging dock: mount an AprilTag on the dock. When battery drops below 20%, the robot navigates to dock proximity, then uses camera visual servo to fine-align and dock.

SECTION 6 — WEB DASHBOARD

## Dashboard Technology Stack

| Layer | Technology | Notes |
| --- | --- | --- |
| Backend | FastAPI (Python) | REST API + WebSocket relay |
| ROS bridge | rosbridge_suite | WebSocket on port 9090 |
| Frontend | React or plain HTML + roslibjs | Connects directly to rosbridge |
| Video stream | web_video_server (ROS pkg) | MJPEG stream from /image_raw |
| Map display | Nav2D or custom Canvas | Subscribes to /map + /amcl_pose |

## Dashboard Features

### Live Data Panel

✅  Battery status (% from ESP32 bus voltage)

✅  Robot position on live map

✅  LiDAR scan status

✅  Camera stream (MJPEG)

✅  Motor RPM and temperature

✅  Current state (Idle / Mapping / Cleaning / Returning)

### Control Buttons

✅  Start cleaning mission

✅  Stop (graceful Nav2 cancel)

✅  Emergency Stop (direct /cmd_vel zero)

✅  Go to Point 1 / Point 2 / Point 3

✅  Return Home

✅  Save current position as waypoint

SECTION 7 — SAFETY SYSTEM

⚠️ SAFETY SYSTEMS MUST BE TESTED BEFORE AUTONOMOUS OPERATION

## Hardware Emergency Stop (ESP32)

The ESP32 operates completely independently of the ROCK 4C+. It continuously polls ultrasonic sensors and can stop the ODrive directly via UART — even if ROS has crashed.

// ESP32 Arduino pseudo-code

if (ultrasonic_distance_cm < STOP_THRESHOLD) {

odrive_uart.write('v 0 0\n');   // velocity = 0

odrive_uart.write('v 1 0\n');

publish_ros_topic('/estop', true);

}

## Software Safety Layers

| Layer | Mechanism | Trigger |
| --- | --- | --- |
| Nav2 costmap | Obstacle inflation radius | LiDAR /scan updates |
| Local planner | DWB velocity constraint | Real-time path following |
| ESP32 hardware stop | Direct ODrive UART command | Ultrasonic < threshold |
| Physical E-stop button | GPIO on ESP32 → ODrive stop | Manual press |
| Low battery action | Return-to-home mission | Battery voltage < 20% |
| ROS watchdog | Systemd restart on node crash | Node exit detected |

SECTION 8 — WEB DASHBOARD DESIGN & LAYOUT

The web dashboard connects to the robot via rosbridge WebSocket on port 9090. It has four main pages accessible from a persistent left sidebar. All pages update in real time.

## Dashboard Pages Overview

| Page | Icon | Purpose | Key ROS Topics |
| --- | --- | --- | --- |
| Main Control | 🏠 | Primary robot control, live map, camera | /map, /amcl_pose, /cmd_vel, /image_raw |
| Mission Planner | 📍 | Set & manage waypoints, run missions | /navigate_to_pose, /waypoints |
| Sensor Monitor | 📡 | Live sensor data, LiDAR scan view | /scan, /imu/data, /ultrasonic |
| System Status | ⚙️ | Battery, motors, ROS node health | /battery, /diagnostics, /odom |

## Page 1 — Main Control

The primary page. Opens by default. Contains live map, camera stream, robot status bar, and quick-action buttons. Sidebar stays visible on all pages.

🤖 Robot Dashboard ● Connected ¦ Battery: 87% ¦ State: Cleaning ¦ IP: 192.168.1.50

🏠 Home 📍 Mission 📡 Sensors ⚙️ System ───────── 🔴 E-STOP ⏹ Stop 🏠 Go Home; 📍 Live Map ───────────────────────────────────── ██████████████████████████████ █ █ █ ████████ ████████ █ █ █ █ █ █ █ █ 🤖→ █ █ █ █ ████████ █ █ █ █ ██████████████████████████████ Pose: x=2.3m y=1.7m θ=45° Map: room1.pgm; 📷 Camera Stream ───────────────────── ▶ LIVE MJPEG Stream 640 × 480 30 fps. Topic: /image_raw. 🔋 Battery 87% Good; ⚡ State Cleaning Active; 🔄 Speed 0.3 m/s Normal; 📡 LiDAR OK Running..

## Page 2 — Mission Planner

Used to define, save, and execute waypoint missions. User clicks 'Add Waypoint' to save the robot's current position, or types coordinates manually. Missions can be saved and reloaded.

🤖 Robot Dashboard ● Connected ¦ Battery: 87% ¦ State: Idle

🏠 Home 📍 Mission 📡 Sensors ⚙️ System ───────── 🔴 E-STOP 🏠 Go Home; 📍 Map — Click to place waypoint ────────────────────────────── ████████████████████████████ █ ① ③ █ █ ↑ ↑ █ █ █ █ 🏠 █ █ ↑ █ █ ② █ ████████████████████████████ ①②③ = Waypoints 🏠 = Home; Mission Waypoints ───────────────────────────── ① Point 1 ✓ Saved x: 1.2m y: 3.4m. ② Point 2 ✓ Saved x: 4.5m y: 1.8m. ③ Point 3 ✓ Saved x: 1.8m y: 5.2m. 🏠 Home Dock x: 0.0m y: 0.0m. + Add Point; ▶ Run Mission; ⏹ Clear All...

## Page 3 — Sensor Monitor

Displays all live sensor data from the robot. LiDAR scan is visualised as a dot-grid. IMU compass heading shown as a bearing. Ultrasonic distances shown as a bar gauge.

🤖 Robot Dashboard ● Connected ¦ Battery: 87% ¦ State: Idle

🏠 Home 📍 Mission 📡 Sensors ⚙️ System ───────── 🔴 E-STOP; 📡 LiDAR Scan /scan ───────────────────── . . . . . . . . . ███████ . . █ █ . . █ ✛ █ . . █ █ . . ███████ . . . . . . . . 360° · 10Hz · 0.15–12m; 🧭 Compass / IMU /imu/data ───────────────────── N ↑ W ← ✛ → E ↓ S Heading: 045° (NE) Roll: 1.2° Pitch: 0.8°; 📏 Ultrasonic /ultrasonic ───────────────────── Front 68cm. Left 22cm. Right 85cm. Rear 12cm. ⚠️ Rear obstacle < 20cm..

## Page 4 — System Status

Shows the health of all ROS nodes, motor telemetry, battery voltage, and network info. Color-coded green/amber/red at a glance.

🤖 Robot Dashboard ● Connected ¦ Battery: 87% ¦ State: Idle

🏠 Home 📍 Mission 📡 Sensors ⚙️ System ───────── 🔴 E-STOP; ROS 2 Node Health Node; Status; Topic / Action; Hz. lidar_node; ● Running; /scan; 10. camera_node; ● Running; /image_raw; 30. slam_toolbox; ● Running; /map, /odom; 5. navigation2; ● Running; /navigate_to_pose; 10. odrive_node; ● Running; /cmd_vel → ODrive; 50. esp32_bridge; ● Running; /imu/data, /ultrasonic; 20. waypoint_mgr; ● Idle; /waypoints; —. rosbridge; ● Running; WebSocket :9090; —. ⚡ Motor Telemetry (ODrive) ──────────────────────────────── Left motor RPM; 142 RPM. Right motor RPM; 139 RPM. Bus voltage; 34.2 V. Motor temp (L); 38°C. Motor temp (R); 41°C. Current (L); 2.3 A. Current (R); 2.5 A.; 🔋 Battery & Network ────────────────────────────────── Battery charge: 87% Voltage: 34.2V Estimated: 2h 15m Robot IP; 192.168.1.50. ROS Domain ID; 0. rosbridge; WS :9090 ● Live. web_video_server; HTTP :8080 ● Live. FastAPI backend; HTTP :8000 ● Live. Uptime; 4h 32m...

SECTION 8B — WEBSOCKET IMPLEMENTATION

## rosbridge WebSocket Data Flow

All communication between the dashboard and robot uses rosbridge_suite. The browser subscribes to ROS topics and calls ROS actions directly via the roslibjs JavaScript library.

| Dashboard Action | Direction | ROS Topic / Action | Message Type |
| --- | --- | --- | --- |
| Live map display | Subscribe ← | /map | nav_msgs/OccupancyGrid |
| Robot pose overlay | Subscribe ← | /amcl_pose | geometry_msgs/PoseWithCovarianceStamped |
| Camera stream | HTTP stream ← | web_video_server :8080 | MJPEG |
| LiDAR scan view | Subscribe ← | /scan | sensor_msgs/LaserScan |
| Compass / IMU | Subscribe ← | /imu/data | sensor_msgs/Imu |
| Ultrasonic data | Subscribe ← | /ultrasonic | std_msgs/Float32MultiArray |
| Battery level | Subscribe ← | /battery_voltage | std_msgs/Float32 |
| Odometry / speed | Subscribe ← | /odom | nav_msgs/Odometry |
| Send navigation goal | Publish → | /navigate_to_pose | nav2_msgs/NavigateToPose |
| Emergency stop | Publish → | /cmd_vel | geometry_msgs/Twist (all zeros) |
| Go to waypoint | Action → | /navigate_to_pose | nav2_msgs/NavigateToPose |
| ROS node health | Subscribe ← | /diagnostics | diagnostic_msgs/DiagnosticArray |

## Quick-Start Code — roslibjs

Paste this in your HTML frontend to connect and subscribe to the live map:

<!-- Include in HTML <head> -->

<script src="https://cdn.jsdelivr.net/npm/roslib/build/roslib.min.js"></script>

// Connect to rosbridge

const ros = new ROSLIB.Ros({ url: 'ws://192.168.1.50:9090' });

ros.on('connection', () => console.log('Connected to robot'));

// Subscribe to live robot pose

const poseTopic = new ROSLIB.Topic({

ros: ros, name: '/amcl_pose',

messageType: 'geometry_msgs/PoseWithCovarianceStamped'

});

poseTopic.subscribe(msg => {

const x = msg.pose.pose.position.x;

const y = msg.pose.pose.position.y;

updateRobotMarkerOnMap(x, y);

});

// Send navigation goal

const navClient = new ROSLIB.ActionClient({

ros: ros, serverName: '/navigate_to_pose',

actionName: 'nav2_msgs/action/NavigateToPose'

});

const goal = new ROSLIB.Goal({

actionClient: navClient,

goalMessage: { pose: { pose: { position: {x:1.2, y:3.4, z:0} }}}

});

goal.send();

SECTION 9 — FUTURE UPGRADES

| Upgrade | Technology | Complexity |
| --- | --- | --- |
| Voice control | Whisper ASR + intent parser | Medium |
| AI object detection | YOLOv8 on camera feed | Medium |
| Physical charging dock | AprilTag + visual servo docking | High |
| Face recognition | face_recognition Python lib | Medium |
| Object delivery | Arm attachment + MoveIt2 | High |
| Multi-room mapping | slam_toolbox map merging | Medium |
| Fleet management | ROS 2 multi-robot namespacing | High |

You are building a proper ROS 2 autonomous mobile robot. Distributed • Headless • Modular • Scalable